Reviewer 1

- Illustration of different lines in Fig. 2A and Fig. 2B should be added to these two subfigures, respectively. Double check such problems across this manuscript.

A legend has been added to Figure 2 and the text size has been enlarged on Figures 2-4.

- The size of word in Figure 4 is too small, which should be revised.

The text size of Figures 2 through 4 has been increased.

- There should be a space between the value and the unit, such as, 490 N. Check and revise across this manuscript.

We’ve used package siunitx with the standard /, space. We checked the document and corrected a few that had a simple space. We can change the default options to a larger space, if needed, but would need approval from MDPI copy-editors.

- A section of Conclusion should be added after the Discussion.

The last two paragraphs have now been organized under a Conclusion section.

- The proposed models are largely empirical. How to demonstrate the versatility of the empirical model for Festo BPAs with different diameters?

This work applies to the 10 and 20 mm diameter muscles, which are the most common. We anticipate that these equations can be used with different coefficients for the other sizes (5mm and 40 mm), but data would need to be collected, and we are not set up to collect that data. We believe that because this provides detailed information on the most common diameter muscles used in the field of robotics, this paper contributes meaningfully to scientific knowledge without the inclusion of data and a fit for the 5 mm and 40 mm muscles and that such findings could be left to future research and publications.

- What is the stiffness of the Festo BPA when a zero pressure is applied? Characterization of such a passive stiffness is demanded.

We agree that the stiffness characterization at zero pressure could be interesting, especially when considering a comparison of passive elements in biological muscles. However, we do not currently have equipment to perform this test, as it would require a unique tensile test system. The work that we have presented provides detailed information for active control of the artificial muscles, while the stiffness at zero pressure could only be found if the actuator is acted on by an outside force. We believe that since our work focuses on active control situations, it contributes meaningfully to scientific knowledge without the inclusion of passive stretch data at zero pressure.

- Experiments are limited to Festo BPAs of two diameters; effects of manufacturing variations are not analyzed. Add necessary discussion on these influences.

We have indeed seen some variation in data that we attribute to manufacturing variations, this is mentioned some in the discussion but was not expanded upon. We have added to the discussion paragraph about this subject and it now reads as follows:

Additionally, manufacturing variations seem to play an important factor in the behavior of these actuators. It remains to be seen how $\epsilon_{620}$ can be known a priori. We still suspect that it is a function of $l_{rest}$, but it might also be a function of other factors such as product batch number or UV light exposure. We found there to be larger variations in the \qtylist{10}{\mm} BPAs than in the \qtylist{20}{\mm} BPAs. Modeling work of ideal McKibben actuators describe how the relationship between contraction and initial length is a function of the total number of twists and the angle of twist in the usable muscle area \citep{chou_measurement_1996}. It is unclear if there are variations in the angle and number of twists in the Festo BPAs due to manufacturing. Uncovering the relationship between $\epsilon_{620}$ and $l_{rest}$ for the Festo DMSP/MAS artificial muscles, if it exists in a meaningful way, will require additional controlled tests. Additionally, it is not clear how the maximum force might change with manufacturing variations. This was reasonably corrected for by normalizing the equations to each individual's maximum contraction, but more nuanced details might emerge with more data collection. Not knowing these relationships a priori means there is still a need for robots and other systems to go through an iterative design stage if it is discovered that the system does not produce the torque or have the RoM that the design team expects. However, the data and models provided here should enable for faster redesigns with fewer additional measurements.

- No analysis of force degradation over cyclic loading or long-term drift is provided. Add these data.

We agree that this would be interesting to look at and an important consideration for long-term use of these artificial muscles. We don’t currently have a test setup for cycling the muscles under load and believe it is outside the scope of this paper, and that data collection of this sort would constitute a whole publication in itself. We have added the following to the discussion section of the paper.

The presented work is capable of predicting force output for 10 mm and 20 mm muscles with different initial muscle lengths, internal pressure, and contraction amount with minimal initial data collection. However, it remains to be seen if these relationships are maintained over the full lifecycle of the muscles. There are not yet any peer reviewed studies investigating the long-term behavior of these artificial muscles, though the Festo datasheet provides some indication of what can be expected [cite]. They report that the 40 mm BPA can complete 1 million cycles at 6000 N (maximum rated load), or 10 million cycles at 4000 N. The service life of the other size BPAs is not reported, however it is mentioned that thermal stressing may reduce the lifespan and they should be inspected every 500,000 cycles for signs of aging [cite]. Future research should look at the behavior of these muscles over time.

- Quantitative comparison with existing models on the same dataset should be conducted to clarify the superiority of proposed model.

The reviewer is correct, we should compare to more than just the Festo tool, and have added comparisons to Sarosi and Martens and Bolan. We have added Table 3 to show a direct comparison of our model with these models as well as a subsection in the discussion that reads as follows.

- Why 620 kPa is chosen as the normalization reference? Whether other pressures can also be employed?

Common building supply pressure here (90 psi). Other pressures could relatively easily be used, like 600 kPa maximum rated pressure for the 20 mm BPA, or 800 kPa which is the maximum rated pressure for the 10 mm BPA. Picking different values in working BPA pressure range would affect coefficients minimally. Doesn’t serve as a true maximum. It allowed us to reduce Equation 8 from 6 coefficients to 3. This discussion is in the paragraph before the Conclusions section.

- Can the proposed model be embedded into real-time control or simulation? Clarify it in Discussion.

Yes. Small equation. Can also be used in lookup table for quick real-time control. We have added a subsection to the Discussion to clarify this.

- Double check the format of references [19] and [20].

Thank you for pointing this out. The references have been corrected.

Reviewer 2

A simple analysis of a product's performance and the difference between the manufacturer's specifications and the individual device.  As a route to characterizing the actuator for use in an eventual product it is acceptable.

- However, it would be informative for the authors to speculate the reasons for the deviations between their model and the manufacturer's.  They identify where but not why.  As experienced users of the devices they might be able to give the reader the benefit of their greater insight.

    - In the 2013 catalogue (https://ftp.festo.com/Public/PNEUMATIC/SOFTWARE_SERVICE/Documentation/2015/EN/DMSP-MAS_EN.PDF), they note that testing was done with BPAs that are 10x the diameter. It is possible model must have a sign with an inverted parameter, because even in some of their documentation they note that force decreases with decreasing length. However, the 100 mm BPA should have much less force than what is shown in the curve. This is our experience and the reason for the testing. They don’t properly account for the end effects of the pressure vessel where end effects dominate: 	Clamped ends that fix diameter but allow axial displacement impose  boundary conditions that force steep gradients in stress and strain near the ends. Near the boundary conditions are where bending and shear deviate from  analytic solutions to a cylindrical model. Therefore, in shorter BPAs, the end effects dominate. The axial length scale of the boundary layer is comparable to the vessel length, so global solution differs substantially from an infinitely long cylinder solution.

- One concern is: "The improved Hill-muscle model of Thelen [22] shows an exponential term for a muscle’s passive force-length property concentric loading."  This model is specifically for aging muscles.  In general the impact of the connective tissue on the curve is a step function.  Do the authors have a more general reference for this?

Added Biomimetic Artificial Muscle Analogy subsection to discussion mentioning the work of Winters and Chou and Hannaford.

- The figures, especially Figure 4, are quite small, the vertical scale reduced.  There are few figures in the paper so I suggest they can afford to take up a little more space.

We have updated these Figures 2 through 4 for ease of readability.

- An observation is that the disadvantage of BPAs are: "maximum tensile force occurs at resting (i.e., uninflated) length (lrest) rather than at an optimal fiber length".  Is the ability of the natural musculoskeletal system to adapt to the required loads and ranges placed upon them, by changing tenson length, sarcomere and fibre numbers, at least as significant problem when trying to mimic the natural system?

To address this, we have added a subsection the end of the discussion that talks more about the analogy between muscles and BPAs. As Winters points out in “Hill-Based Muscle Models: A Systems Engineering Perspective,” even the Hill model can sometimes be seen as either too much of a simplification or as overly complex.